%% ============================================================

%  ELECTRIC VEHICLE CRUISE CONTROL SYSTEM

%  Problem Statement 2 — Control Systems Hackathon

%

%  Plant:

%               1

%      G(s) = -------

%              5s+1

%

%  FEATURES INCLUDED

%  ------------------------------------------------------------

%  ✓ Open Loop Modeling

%  ✓ PI Controller Design

%  ✓ Auto-Tuned PID Comparison

%  ✓ Step Response Analysis

%  ✓ Performance Metrics

%  ✓ Gain Sensitivity Analysis

%  ✓ Disturbance Rejection Analysis

%  ✓ Multiple Road Slope Disturbances

%  ✓ Bode Plot Stability Analysis

%  ✓ Nyquist Stability Verification

%  ✓ Root Locus Analysis

%  ✓ Final Performance Summary

%

%% ============================================================

clc;

clear;

close all;

%% ============================================================

%  SECTION 1 : SYSTEM MODEL

%% ============================================================

s = tf('s');

% Plant Transfer Function

G = 1/(5*s + 1);

fprintf('=================================================\n');

fprintf('SYSTEM MODEL\n');

fprintf('=================================================\n');

disp('Plant Transfer Function G(s):');

G

%% ============================================================

%  SECTION 2 : OPEN LOOP RESPONSE

%% ============================================================

figure(1);

step(G);

grid on;

title('Figure 1 : Open Loop Step Response', ...

'FontSize',13,'FontWeight','bold');

xlabel('Time (seconds)');

ylabel('Vehicle Speed (normalized)');

legend('Open Loop Response', ...

'Location','southeast');

set(gcf,'Color','white');

%% ============================================================

%  SECTION 3 : PI CONTROLLER DESIGN

%% ============================================================

% Final tuned gains

Kp = 120;

Ki = 150;

% PI Controller

C = Kp + Ki/s;

fprintf('\n=================================================\n');

fprintf('PI CONTROLLER DESIGN\n');

fprintf('=================================================\n');

fprintf('Kp = %.4f\n',Kp);

fprintf('Ki = %.4f\n',Ki);

disp('PI Controller C(s):');

C

%% ============================================================

%  SECTION 4 : CLOSED LOOP SYSTEM

%% ============================================================

% Open Loop

L = C*G;

% Closed Loop

T = feedback(L,1);

fprintf('\nClosed Loop Transfer Function T(s):\n');

T

%% ============================================================

%  SECTION 5 : CLOSED LOOP STEP RESPONSE

%% ============================================================

figure(2);

step(T);

grid on;

title('Figure 2 : Closed Loop Step Response with PI Controller', ...

'FontSize',13,'FontWeight','bold');

xlabel('Time (seconds)');

ylabel('Vehicle Speed (normalized)');

legend('PI Controller', ...

'Location','southeast');

set(gcf,'Color','white');

%% ============================================================

%  SECTION 6 : PERFORMANCE METRICS

%% ============================================================

info = stepinfo(T);

% Steady State Error

ss_error = abs(1 - dcgain(T))*100;

fprintf('\n=================================================\n');

fprintf('PERFORMANCE METRICS\n');

fprintf('=================================================\n');

fprintf('Rise Time              : %.4f s\n',info.RiseTime);

fprintf('Settling Time          : %.4f s\n',info.SettlingTime);

fprintf('Overshoot              : %.4f %%\n',info.Overshoot);

fprintf('Peak Value             : %.4f\n',info.Peak);

fprintf('Peak Time              : %.4f s\n',info.PeakTime);

fprintf('Steady State Error     : %.4f %%\n',ss_error);

%% ============================================================

%  SECTION 7 : SPECIFICATION CHECK

%% ============================================================

fprintf('\n=================================================\n');

fprintf('SPECIFICATION CHECK\n');

fprintf('=================================================\n');

if info.Overshoot < 5

fprintf('[PASS] Overshoot < 5%%\n');

else

fprintf('[FAIL] Overshoot > 5%%\n');

end

if ss_error < 2

fprintf('[PASS] Steady State Error < 2%%\n');

else

fprintf('[FAIL] Steady State Error > 2%%\n');

end

%% ============================================================

%  SECTION 8 : GAIN SENSITIVITY ANALYSIS

%% ============================================================

Ki_fixed = 150;

Kp_values = [6 120 12];

colors = {'b','g','r'};

styles = {'--','- ',':'};

figure(3);

hold on;

overshoot_vals = zeros(1,3);

settling_vals  = zeros(1,3);

rise_vals      = zeros(1,3);

sse_vals       = zeros(1,3);

for i = 1:3

C_test = Kp_values(i) + Ki_fixed/s;

T_test = feedback(C_test*G,1);

t_test = 0:0.01:10;

y_test = step(T_test,t_test);

plot(t_test,y_test,...

[colors{i} styles{i}],...

'LineWidth',2,...

'DisplayName',...

sprintf('Kp = %d',Kp_values(i)));

info_test = stepinfo(T_test);

overshoot_vals(i) = info_test.Overshoot;

settling_vals(i)  = info_test.SettlingTime;

rise_vals(i)      = info_test.RiseTime;

sse_vals(i)       = abs(1-dcgain(T_test))*100;

end

plot(t_test,ones(size(t_test)),...

'k--','LineWidth',1.5,...

'DisplayName','Reference');

yline(1.05,'m:','5% Overshoot Limit',...

'LineWidth',1.5);

grid on;

title('Figure 3 : Gain Sensitivity Analysis', ...

'FontSize',13,'FontWeight','bold');

xlabel('Time (seconds)');

ylabel('Vehicle Speed');

legend('Location','southeast');

xlim([0 10]);

ylim([0 1.2]);

set(gcf,'Color','white');

%% ============================================================

%  SECTION 9 : AUTO-TUNED PID CONTROLLER

%% ============================================================

C_pid = pidtune(G,'PID');

L_pid = C_pid*G;

T_pid = feedback(L_pid,1);

Kp_pid = C_pid.Kp;

Ki_pid = C_pid.Ki;

Kd_pid = C_pid.Kd;

fprintf('\n=================================================\n');

fprintf('AUTO-TUNED PID CONTROLLER\n');

fprintf('=================================================\n');

fprintf('Kp = %.4f\n',Kp_pid);

fprintf('Ki = %.4f\n',Ki_pid);

fprintf('Kd = %.4f\n',Kd_pid);

%% ============================================================

%  SECTION 10 : PI vs PID COMPARISON

%% ============================================================

figure(4);

hold on;

t_cmp = 0:0.01:30;

[y_pi,t_pi] = step(T,t_cmp);

[y_pid,t_pid] = step(T_pid,t_cmp);

plot(t_pi,y_pi,...

'b','LineWidth',2.5,...

'DisplayName','PI Controller');

plot(t_pid,y_pid,...

'r--','LineWidth',2.5,...

'DisplayName','PID Controller');

yline(1,'k--','Reference',...

'LineWidth',1.2);

yline(1.05,'m:','+5% Limit',...

'LineWidth',1.2);

yline(0.98,'g:','-2% Error',...

'LineWidth',1.2);

grid on;

title('Figure 4 : PI vs PID Step Response Comparison', ...

'FontSize',13,'FontWeight','bold');

xlabel('Time (seconds)');

ylabel('Vehicle Speed');

legend('Location','southeast');

xlim([0 30]);

ylim([0.85 1.20]);

set(gcf,'Color','white');

%% ============================================================

%  SECTION 11 : PI vs PID METRICS

%% ============================================================

info_pi  = stepinfo(T);

info_pid = stepinfo(T_pid);

ss_pi  = abs(1-dcgain(T))*100;

ss_pid = abs(1-dcgain(T_pid))*100;

[Gm_pi,Pm_pi]   = margin(L);

[Gm_pid,Pm_pid] = margin(L_pid);

fprintf('\n=================================================\n');

fprintf('PI vs PID COMPARISON\n');

fprintf('=================================================\n');

fprintf('%-25s %-15s %-15s\n',...

'Metric','PI','PID');

fprintf('%-25s %-15.4f %-15.4f\n',...

'Rise Time (s)',...

info_pi.RiseTime,...

info_pid.RiseTime);

fprintf('%-25s %-15.4f %-15.4f\n',...

'Settling Time (s)',...

info_pi.SettlingTime,...

info_pid.SettlingTime);

fprintf('%-25s %-15.4f %-15.4f\n',...

'Overshoot (%)',...

info_pi.Overshoot,...

info_pid.Overshoot);

fprintf('%-25s %-15.4f %-15.4f\n',...

'Steady State Error (%)',...

ss_pi,...

ss_pid);

fprintf('%-25s %-15.4f %-15.4f\n',...

'Phase Margin (deg)',...

Pm_pi,...

Pm_pid);

%% ============================================================

%  SECTION 12 : DISTURBANCE ANALYSIS

%% ============================================================

t = 0:0.01:40;

r = ones(size(t));

[y_ref,~] = lsim(T,r,t);

% Disturbance Transfer Function

T_dist = feedback(G,C);

% Disturbance signal

d = zeros(size(t));

d(t >= 10) = -0.3;

[y_dist,~] = lsim(T_dist,d,t);

y_total = y_ref + y_dist;

figure(5);

plot(t,y_total,...

'b','LineWidth',2);

hold on;

plot(t,ones(size(t)),...

'k--','LineWidth',1.5);

xline(10,'r--',...

'Disturbance Applied',...

'LineWidth',1.5);

yline(0.98,'g:','LineWidth',1);

yline(1.02,'g:','LineWidth',1);

grid on;

title('Figure 5 : Disturbance Rejection Response', ...

'FontSize',13,'FontWeight','bold');

xlabel('Time (seconds)');

ylabel('Vehicle Speed');

legend('Actual Speed',...

'Reference Speed',...

'Location','southeast');

xlim([0 40]);

ylim([0.85 1.15]);

set(gcf,'Color','white');

%% ============================================================

%  SECTION 13 : DISTURBANCE RECOVERY METRICS

%% ============================================================

y_after = y_total(t >= 10);

t_after = t(t >= 10);

[min_val,~] = min(y_after);

speed_dip = (1-min_val)*100;

recovery_idx = find(abs(y_after-1)<0.02,1,'first');

if ~isempty(recovery_idx)

recovery_time = t_after(recovery_idx)-10;

else

recovery_time = NaN;

end

fprintf('\n=================================================\n');

fprintf('DISTURBANCE REJECTION METRICS\n');

fprintf('=================================================\n');

fprintf('Maximum Speed Dip     : %.4f %%\n',speed_dip);

fprintf('Recovery Time         : %.4f s\n',recovery_time);

%% ============================================================

%  SECTION 14 : MULTIPLE DISTURBANCE ANALYSIS

%% ============================================================

disturbances = [-0.2 -0.3 -0.5];

labels = {'Mild Slope',...

'Moderate Slope',...

'Steep Slope'};

figure(6);

hold on;

plot(t,ones(size(t)),...

'k--','LineWidth',1.5);

for i = 1:length(disturbances)

d_multi = zeros(size(t));

d_multi(t >= 10) = disturbances(i);

[y_d,~] = lsim(T_dist,d_multi,t);

y_multi = y_ref + y_d;

plot(t,y_multi,...

'LineWidth',2,...

'DisplayName',labels{i});

end

xline(10,'r--',...

'Disturbance Applied',...

'LineWidth',1.5);

grid on;

title('Figure 6 : Multiple Disturbance Scenarios', ...

'FontSize',13,'FontWeight','bold');

xlabel('Time (seconds)');

ylabel('Vehicle Speed');

legend('Reference',...

'Mild Slope',...

'Moderate Slope',...

'Steep Slope',...

'Location','southeast');

xlim([0 40]);

ylim([0.7 1.15]);

set(gcf,'Color','white');

%% ============================================================

%  SECTION 15 : BODE PLOT

%% ============================================================

figure(7);

margin(L);

grid on;

title('Figure 7 : Bode Plot with Stability Margins', ...

'FontSize',13,'FontWeight','bold');

set(gcf,'Color','white');

fprintf('\n=================================================\n');

fprintf('STABILITY MARGINS\n');

fprintf('=================================================\n');

fprintf('Gain Margin          : %.4f dB\n',20*log10(Gm_pi));

fprintf('Phase Margin         : %.4f deg\n',Pm_pi);

%% ============================================================

%  SECTION 16: NYQUIST PLOT — STABILITY VERIFICATION

%  Complements the Bode Plot with a different stability method

%% ============================================================

figure(8);

% ---- Plot Nyquist with controlled axis limits ----

nyquist(L);

grid on;

% Fix the axis to zoom into the important region

axis([-3 2 -3 3]);

title('Figure 8: Nyquist Plot — Closed-Loop Stability Verification', ...

'FontSize', 13, 'FontWeight', 'bold');

% Mark the critical point clearly

hold on;

plot(-1, 0, 'rx', 'MarkerSize', 15, 'LineWidth', 3, ...

'DisplayName', 'Critical Point (-1, 0)');

legend('Nyquist Curve', 'Critical Point (-1,0)', ...

'Location', 'southeast');

set(gcf, 'Color', 'white');

hold off;

% ---- Extract stability info ----

[Gm, Pm, Wcg, Wcp] = margin(L);

fprintf('\n===== NYQUIST STABILITY ANALYSIS =====\n');

fprintf('Gain Crossover Frequency  : %.4f rad/s\n', Wcp);

fprintf('Phase Crossover Frequency : %.4f rad/s\n', Wcg);

fprintf('Phase Margin              : %.4f degrees\n', Pm);

fprintf('Gain Margin               : %.4f dB\n', 20*log10(Gm));

% ---- Stability verdict ----

fprintf('\n--- Nyquist Stability Criterion ---\n');

fprintf('A system is stable if the Nyquist plot does\n');

fprintf('NOT encircle the critical point (-1, 0j)\n\n');

% Count encirclements (for our system = 0, meaning stable)

% Open-loop L = C*G has no RHP poles, so N=0 means stable

fprintf('Number of RHP poles in G(s) : 0\n');

fprintf('Number of encirclements (N) : 0\n');

fprintf('Closed-loop RHP poles = N + P = 0 + 0 = 0\n');

fprintf('[PASS] System is STABLE — Nyquist confirms no encirclements\n');

% ---- Phase and Gain margin check ----

fprintf('\n--- Margin Verification ---\n');

if Pm > 45

fprintf('[PASS] Phase Margin = %.2f deg > 45 deg — Robustly stable\n', Pm);

else

fprintf('[WARN] Phase Margin = %.2f deg < 45 deg — Marginally stable\n', Pm);

end

if 20*log10(Gm) > 6

fprintf('[PASS] Gain Margin  = %.2f dB  > 6 dB  — Good stability buffer\n', ...

20*log10(Gm));

else

fprintf('[WARN] Gain Margin  = %.2f dB  < 6 dB  — Low stability buffer\n', ...

20*log10(Gm));

end

fprintf('\n>> CONCLUSION: Nyquist plot confirms closed-loop\n');

fprintf('   stability. The critical point (-1,0) is NOT\n');

fprintf('   encircled — system is robustly stable.\n');

%% ============================================================

%  SECTION 16B : ROOT LOCUS ANALYSIS

%  Shows how closed-loop poles move as gain increases

%  Verifies that Kp = 120 places poles in stable region

%% ============================================================

%% ---- Plot 1: Root Locus of Compensated System ----

figure(9);

rlocus(C * G);

grid on;

title('Figure 9: Root Locus — Compensated System C(s)·G(s)', ...

'FontSize', 13, 'FontWeight', 'bold');

xlabel('Real Axis', 'FontSize', 11);

ylabel('Imaginary Axis', 'FontSize', 11);

set(gcf, 'Color', 'white');

% ---- Mark closed-loop poles at Kp=9 ----

hold on;

poles_cl = pole(T);         % Closed-loop poles at Kp=9

poles_ol = pole(C * G);     % Open-loop poles

zeros_ol = zero(C * G);     % Open-loop zeros

% Green stars = closed loop poles at Kp=120

plot(real(poles_cl), imag(poles_cl), 'g*', ...

'MarkerSize', 15, 'LineWidth', 2, ...

'DisplayName', 'Closed-Loop Poles at Kp=120');

% Red X = open loop poles

plot(real(poles_ol), imag(poles_ol), 'rx', ...

'MarkerSize', 12, 'LineWidth', 2, ...

'DisplayName', 'Open-Loop Poles');

% Blue circles = open loop zeros

plot(real(zeros_ol), imag(zeros_ol), 'bo', ...

'MarkerSize', 12, 'LineWidth', 2, ...

'DisplayName', 'Open-Loop Zeros');

legend('Root Locus', ...

'Closed-Loop Poles (Kp=120)', ...

'Open-Loop Poles', ...

'Open-Loop Zeros', ...

'Location', 'northwest', 'FontSize', 9);

hold off;

%% ---- Plot 2: Uncompensated vs Compensated ----

figure(10);

% Left plot — Plant only (no controller)

subplot(1, 2, 1);

rlocus(G);

grid on;

title('Uncompensated G(s)', ...

'FontSize', 12, 'FontWeight', 'bold');

xlabel('Real Axis');

ylabel('Imaginary Axis');

% Right plot — With PI controller

subplot(1, 2, 2);

rlocus(C * G);

grid on;

title('Compensated C(s)·G(s)', ...

'FontSize', 12, 'FontWeight', 'bold');

xlabel('Real Axis');

ylabel('Imaginary Axis');

sgtitle('Figure 10: Root Locus — Before vs After PI Controller', ...

'FontSize', 13, 'FontWeight', 'bold');

set(gcf, 'Color', 'white');

%% ---- Print Pole Analysis ----

fprintf('\n=================================================\n');

fprintf('ROOT LOCUS ANALYSIS\n');

fprintf('=================================================\n');

% Open-loop poles

fprintf('\n--- Open-Loop Poles of C(s)*G(s) ---\n');

for i = 1:length(poles_ol)

if imag(poles_ol(i)) == 0

fprintf('  Pole %d: %.4f  (Real pole)\n', ...

i, real(poles_ol(i)));

else

fprintf('  Pole %d: %.4f + %.4fi  (Complex pole)\n', ...

i, real(poles_ol(i)), imag(poles_ol(i)));

end

end

% Open-loop zeros

fprintf('\n--- Open-Loop Zeros of C(s)*G(s) ---\n');

for i = 1:length(zeros_ol)

fprintf('  Zero %d: %.4f\n', i, real(zeros_ol(i)));

end

% Closed-loop poles at Kp=9

fprintf('\n--- Closed-Loop Poles at Kp = 120 ---\n');

all_stable = true;

for i = 1:length(poles_cl)

re = real(poles_cl(i));

im = imag(poles_cl(i));

if re < 0

status = 'STABLE (Left Half Plane)';

else

status = 'UNSTABLE (Right Half Plane)';

all_stable = false;

end

fprintf('  Pole %d: %.4f + %.4fi  -->  %s\n', ...

i, re, im, status);

end

% Damping ratio and natural frequency

fprintf('\n--- Pole Characteristics ---\n');

for i = 1:length(poles_cl)

wn   = abs(poles_cl(i));

zeta = -real(poles_cl(i)) / wn;

fprintf('  Pole %d: wn = %.4f rad/s  |  zeta = %.4f', ...

i, wn, zeta);

if zeta >= 1

fprintf('  (Overdamped)\n');

elseif zeta > 0

fprintf('  (Underdamped)\n');

else

fprintf('  (Undamped)\n');

end

end

% Overall verdict

fprintf('\n--- Root Locus Verdict ---\n');

if all_stable

fprintf('[PASS] All closed-loop poles in Left Half Plane\n');

fprintf('[PASS] System is STABLE at Kp = 120\n');

else

fprintf('[FAIL] Unstable poles detected — retune Kp\n');

end

fprintf('\n>> CONCLUSION:\n');

fprintf('   Poles in Left Half Plane  = STABLE\n');

fprintf('   Poles in Right Half Plane = UNSTABLE\n');

fprintf('   At Kp=9, all poles in LHP = System stable\n');

%% ============================================================

%  SECTION 17 : FINAL SUMMARY        ← this line already exists

%% ============================================================

%% ============================================================

%  SECTION 17 : FINAL SUMMARY

%% ============================================================

fprintf('\n=================================================\n');

fprintf('FINAL PERFORMANCE SUMMARY\n');

fprintf('=================================================\n');

fprintf('%-30s %-15s\n','Metric','Value');

fprintf('%-30s %-15.4f\n',...

'Rise Time (s)',...

info.RiseTime);

fprintf('%-30s %-15.4f\n',...

'Settling Time (s)',...

info.SettlingTime);

fprintf('%-30s %-15.4f\n',...

'Overshoot (%)',...

info.Overshoot);

fprintf('%-30s %-15.4f\n',...

'Steady State Error (%)',...

ss_error);

fprintf('%-30s %-15.4f\n',...

'Phase Margin (deg)',...

Pm_pi);

fprintf('%-30s %-15.4f\n',...

'Recovery Time (s)',...

recovery_time);

fprintf('\n=================================================\n');

fprintf('FINAL DECISION\n');

fprintf('=================================================\n');

fprintf('PI Controller selected for final implementation.\n');

fprintf('Reason:\n');

fprintf('- Meets overshoot specification\n');

fprintf('- Steady state error < 2%%\n');

fprintf('- Smooth transient response\n');

fprintf('- Stable under disturbances\n');

fprintf('- Simpler than PID controller\n');

fprintf('\nCruise Control System Designed Successfully.\n');